## I. CƠ BẢN – TRUY CẬP & THÔNG TIN HỆ THỐNG

# Đăng nhập CLI

enable

show version

show inventory

show summary

show ap-name <AP_name>

show aps

show ap debug system-status

show clock

show log system

show log security

## II. CẤU HÌNH HỆ THỐNG (System Settings)

configure terminal

hostname AP-Lobby

clock timezone ICT 7

ntp server 192.168.1.1

management-vlan 10

ip default-gateway 192.168.10.1

write memory

## III. QUẢN LÝ VLAN & GIAO DIỆN MẠNG

interface gigabitethernet 0/0/0

no shutdown

switchport mode trunk

switchport trunk allowed vlan 10,20,30

exit

show vlan

show interface brief

## IV. CẤU HÌNH SSID / WLAN

wlan ssid-profile Office-WiFi

essid "Office-WiFi"

opmode wpa2-aes

wpa-passphrase MySecurePass123

vlan 10

max-authentication-failures 3

exit

wlan virtual-ap Office-WiFi-VAP

ssid-profile Office-WiFi

vlan 10

type employee

exit

show wlan ssid

show wlan virtual-ap

## V. RADIUS / AAA / BẢO MẬT XÁC THỰC

aaa authentication-server radius RADIUS1

host 192.168.10.5

key MySharedSecret

exit

aaa profile Corp-AAA

authentication-server radius RADIUS1

initial-role authenticated

exit

show aaa authentication

show aaa radius statistics

## VI. ROLE & POLICY (Access Control)

user-role employee

access-rule allowall

vlan 10

exit

user-role guest

access-rule denyall

captive-portal-profile GuestPortal

vlan 20

exit

show rights

show user-role

## VII. MESH, RAP, TUNNEL

ap-name Mesh01

mesh-role mesh-point

mesh-cluster-name CampusMesh

mesh-cluster-key MeshSecret123

exit

ap-name RAP01

remote-ap enable

controller-ip 203.113.10.10

exit

show ap mesh topology

show ap remote

## VIII. MONITORING & KIỂM TRA KẾT NỐI

ping 8.8.8.8

ping 192.168.1.1 source vlan 10

traceroute 8.8.8.8

show ap association

show clients

show station-table

show ap debug radio-stats

## IX. RADIO & RF OPTIMIZATION

rf arm-profile default

min-tx-power 6

max-tx-power 18

band-steering-mode prefer-5ghz

exit

show ap arm rf-summary

show ap radio

show ap debug radio-stats

## X. BẢO MẬT & IDS/IPS

ids general-profile default

protect-ssid enable

ap-detect enable

exit

show ids summary

show wips rogue-ap

show wips rogue-sta

## XI. SAO LƯU, KHỞI ĐỘNG & CẬP NHẬT

copy running-config tftp tftp://192.168.1.100/ap_backup.cfg

copy flash: tftp://192.168.1.100/ap_image_backup

reload

show image version

upgrade-image tftp tftp://192.168.1.100/aruba_ap_image.img

## XII. LỆNH NÂNG CAO & DEBUG

show datapath session table

show datapath stats

show ap debug counters

show ap debug system-status

debug user <MAC>

debug wlan auth

show tech-support

## XIII. CLI TRÊN ARUBA INSTANT (IAP)

show running-config

show uptime

show ap debug cloud

show log wireless

show ap debug capwap

## XIV. CLI TRÊN ARUBA CONTROLLER (ARUBAOS)

show ap database

show ap active

show ap config

ap reboot ap-name <name>

show user-table