# TỔNG HỢP LỆNH ARUBAOS-CX (CƠ BẢN → NÂNG CAO)

## I. LỆNH CƠ BẢN & QUẢN TRỊ HỆ THỐNG

enable

configure terminal

hostname SW-Core01

clock timezone ICT 7

clock set 12:30:00 2025-11-04

show version

show system

show interface brief

show running-config

show startup-config

copy running-config startup-config

copy running-config tftp tftp://192.168.1.10/config.cfg

erase startup-config

reload

## II. CẤU HÌNH VLAN & IP INTERFACE

vlan 10

name "User"

exit

vlan 20

name "Server"

exit

interface vlan 10

ip address 192.168.10.1/24

interface vlan 20

ip address 192.168.20.1/24

ip default-gateway 192.168.1.1

show vlan

## III. INTERFACE / TRUNK / LAG (LACP)

interface 1/1/1

description "Uplink to Core"

no shutdown

vlan trunk allowed 10,20,30

vlan trunk native 10

spanning-tree bpdu-guard

exit

interface lag 1

no shutdown

lacp mode active

vlan trunk allowed all

exit

interface 1/1/1-1/1/2

lag 1

show interfaces status

show lacp interfaces

## IV. SPANNING TREE (STP / RSTP)

spanning-tree mode rstp

spanning-tree priority 4096

spanning-tree vlan 10-20 root primary

show spanning-tree

## V. ROUTING (STATIC, OSPF, VRF)

ip routing

ip route 0.0.0.0/0 192.168.1.1

show ip route

router ospf 1

router-id 1.1.1.1

area 0

interface vlan 10

interface vlan 20

show ospf neighbor

show ospf interface

## VI. EIGRP & BGP (AOS-CX hỗ trợ qua VRF & Service)

router bgp 65001

router-id 10.1.1.1

neighbor 10.1.1.2 remote-as 65001

neighbor 10.1.1.2 update-source loopback0

network 10.10.10.0/24

exit

show bgp summary

show bgp neighbors

## VII. BẢO MẬT PORT & ACCESS

interface 1/1/3

shutdown

exit

interface 1/1/4

no shutdown

port-access security

port-access max-allowed 2

port-access sticky

port-access violation-action shutdown

show port-access clients

## VIII. DHCP RELAY, SNOOPING

interface vlan 10

ip helper-address 192.168.1.5

dhcpv4-snooping

dhcpv4-snooping vlan 10,20

interface 1/1/24

dhcpv4-snooping trust

show dhcpv4-snooping statistics

## IX. QoS – QUALITY OF SERVICE

qos trust dscp

interface 1/1/1

qos trust cos

qos schedule-profile strict-priority

show qos queue

## X. ACL – ACCESS CONTROL LIST

acl ipv4 BLOCK_HTTP

10 deny tcp any any eq 80

20 permit ip any any

exit

interface vlan 10

apply access-group BLOCK_HTTP in

show access-list

## XI. SNMP, SYSLOG, NTP, AAA

snmp-server community public ro

snmp-server community private rw

logging 192.168.1.100

logging facility local7

ntp server 192.168.1.1

ntp enable

aaa authentication login default local

username admin password <your_password> role admin

show management

## XII. VSX (Virtual Switching Extension)

Dùng cho switch core Aruba 6400, 8320, 8400

vsx

system-mac 00:00:5e:00:01:01

inter-switch-link lag 1

keepalive peer 10.0.0.2 source 10.0.0.1 vrf default

role primary

exit

show vsx status

## XIII. CHẨN ĐOÁN & MONITORING

show interface counters

show interface errors

show logging

show tech

ping 8.8.8.8 vrf default

traceroute 8.8.8.8