# TỔNG HỢP LỆNH ARUBAOS-SWITCH (CƠ BẢN → NÂNG CAO)

## I. LỆNH CƠ BẢN & QUẢN TRỊ HỆ THỐNG

enable
configure terminal
hostname SW-Access01
clock set 12:30:00 04 Nov 2025

write memory
show running-config
show startup-config
copy running-config tftp 192.168.1.100 filename
copy tftp startup-config 192.168.1.100 filename
erase startup-config
reload

show version
show system
show flash
show modules
show time
show interfaces brief

## II. CẤU HÌNH VLAN & TRUNK

vlan 10
   name "Sales"
   untagged 1-10
   tagged 24
   exit
vlan 20
   name "IT"
   untagged 11-20
   exit
show vlan

interface 24
   name "Uplink"
   tagged vlan 10,20,30
   exit

vlan 10
   ip address 192.168.10.1 255.255.255.0
vlan 20
   ip address 192.168.20.1 255.255.255.0
ip default-gateway 192.168.1.1

## III. STP (Spanning Tree Protocol)

spanning-tree
spanning-tree vlan 1-20 priority 4096
spanning-tree 1-20 admin-edge-port
spanning-tree 1-20 bpdu-protection
show spanning-tree

## IV. LINK AGGREGATION (LACP)

trunk 1-2 trk1 lacp
vlan 10
   tagged trk1
show trunk

## V. ROUTING (STATIC, RIP, OSPF)

ip routing
ip route 0.0.0.0 0.0.0.0 192.168.1.1

router rip
   version 2
   network 192.168.10.0
   network 192.168.20.0
   exit
show ip rip

router ospf
   router-id 1.1.1.1
   area 0
      network 192.168.10.0 0.0.0.255
      network 192.168.20.0 0.0.0.255
      exit
   exit
show ip ospf neighbor

## VI. BẢO MẬT PORT & TRUY CẬP

interface 1
   port-security
   port-security max-mac-count 2
   port-security learn-mode sticky
   port-security violation-action send-alarm
show port-security

interface 11-24
   disable

crypto key generate ssh rsa bits 2048
ip ssh
password manager
password operator

## VII. DHCP RELAY / SNOOPING

vlan 10
   ip helper-address 192.168.1.10

dhcp-snooping
dhcp-snooping vlan 10,20
interface 24
   dhcp-snooping trust
show dhcp-snooping

## VIII. QoS (Quality of Service)

qos trust dscp
qos priority 7
interface 1-10
   qos trust cos

## IX. SNMP, SYSLOG, NTP

snmp-server community public operator
snmp-server community private manager
logging 192.168.1.50
logging severity info
ntp server 192.168.1.10
ntp enable
show ntp

## X. STACKING (VSF)

vsf enable domain 1
vsf member 1 link 1
vsf member 2 link 2
vsf renumber-to 1
vsf member 1 priority 255
vsf member 2 priority 128

## XI. GIÁM SÁT & CHẨN ĐOÁN

show interfaces counters
show interfaces status
show interfaces errors
show system-information
show power-over-ethernet brief
ping 192.168.1.1
traceroute 8.8.8.8
show log
show tech

## XII. ACL (Access Control List)

ip access-list extended BLOCK_HTTP
   deny tcp any any eq 80
   permit ip any any
   exit
vlan 10
   ip access-group BLOCK_HTTP in
show access-list