### I. LỆNH CƠ BẢN – KIỂM TRA & QUẢN LÝ

enable

show version

show inventory

show ap summary

show ap name <AP_Name>

show interface summary

show ip interface brief

show running-config

show startup-config

copy running-config startup-config

reload

### II. CẤU HÌNH HỆ THỐNG CƠ BẢN

configure terminal

hostname AP-Lobby

clock timezone ICT 7

clock set 12:00:00 4 NOV 2025

service password-encryption

banner motd ^C Authorized Access Only ^C

### III. CẤU HÌNH MẠNG & VLAN

(áp dụng cho AP EWC hoặc AP Standalone)

interface GigabitEthernet0

description Uplink to Switch

switchport mode trunk

switchport trunk allowed vlan 10,20,30

no shutdown

exit

interface vlan 10

ip address 192.168.10.2 255.255.255.0

no shutdown

ip default-gateway 192.168.10.1

show vlan brief

show interface trunk

### IV. CẤU HÌNH SSID / WLAN (EWC hoặc WLC CLI)

wlan Office-WiFi 1 Office-WiFi

client vlan 10

security wpa psk set-key ascii 0 MySecurePass123

no shutdown

exit

wlan Guest 2 Guest

client vlan 20

security web-auth

no shutdown

exit

show wlan summary

show wlan id 1

### V. AAA / RADIUS / TACACS

aaa new-model

radius server RADIUS1

address ipv4 192.168.1.10 auth-port 1812 acct-port 1813

key MyRadiusKey

exit

aaa group server radius RADIUS-GROUP

server name RADIUS1

exit

aaa authentication dot1x default group RADIUS-GROUP

show radius statistics

### VI. CAPWAP / JOIN CONTROLLER

(áp dụng cho Lightweight AP)

capwap ap hostname AP-Lobby

capwap ap ip address 192.168.10.2 255.255.255.0

capwap ap ip default-gateway 192.168.10.1

capwap ap primary-base WLC1 192.168.10.10

capwap ap controller ip address 192.168.10.10

capwap ap restart

show capwap client config

show capwap client rcb

### VII. RADIO & RF CONFIGURATION

show ap name <AP_Name> config general

show ap name <AP_Name> dot11 5ghz summary

show ap name <AP_Name> dot11 24ghz summary

ap name <AP_Name>

dot11 5ghz

channel 149

power 3

dot11 24ghz

channel 6

power 2

exit

show ap auto-rf 802.11a summary

show ap auto-rf 802.11b summary

### VIII. QoS / WMM / CLIENT MANAGEMENT

wlan Office-WiFi

qos trust dscp

qos policy voice optimize

exit

show client summary

show client detail <mac-address>

clear client <mac-address>

### IX. ACL / POLICY

ip access-list extended BLOCK_HTTP

deny tcp any any eq 80

permit ip any any

exit

wlan Office-WiFi

ipv4 acl BLOCK_HTTP in

exit

show access-lists

### X. MONITORING & DEBUG

show logging

show logging | include error

show controllers dot11Radio 0

show controllers dot11Radio 1

debug capwap events enable

debug capwap errors enable

debug dot11 mgmt enable

debug client <mac-address>

### XI. TROUBLESHOOTING

ping 8.8.8.8

ping 192.168.10.1 source vlan 10

traceroute 8.8.8.8

show ip route

show arp

show tech-support

### XII. SOFTWARE & IMAGE MANAGEMENT

show version

show flash:

dir flash:

archive download-sw /overwrite /reload tftp://192.168.1.10/ap_image.bin

copy running-config tftp tftp://192.168.1.10/ap_backup.cfg

copy flash: tftp://192.168.1.10/ap_log.txt

### XIII. EWC (Embedded Wireless Controller – C9800 Series trên C9000)

show wireless management trustpoint

show wireless management interface

show wireless summary

show ap summary

show wlan summary

show wireless client summary

show wireless client mac-address <MAC>

show wireless mobility summary

show wireless profile policy all

### XIV. HIGH AVAILABILITY (HA / SSO / REDUNDANCY)

redundancy

mode sso

main-cpu

standby console enable

exit

show redundancy summary

show redundancy state

### XV. LỆNH DEBUG & CHẨN ĐOÁN NÂNG CAO

debug dot11 trace print enable

debug dot11 error enable

debug client events enable

debug dhcp message enable

debug capwap events enable

debug mobility handoff enables

### XVI. KIỂM TRA HIỆU NĂNG RADIO

show ap dot11 5ghz summary

show ap dot11 24ghz summary

show ap auto-rf dot11 5ghz

show ap channel

show ap power

### XVII. KHỞI ĐỘNG & RESET

write erase

delete flash:vlan.dat

reload

capwap ap factory-reset