# CẤU HÌNH EIGRP, OSPF VÀ iBGP TRÊN CISCO

## I. CẤU HÌNH EIGRP

router eigrp 100
 network 192.168.10.0 0.0.0.255
 network 192.168.20.0 0.0.0.255
 no auto-summary

router eigrp 100
 passive-interface default
 no passive-interface gig0/0

show ip eigrp neighbors
show ip eigrp topology
show ip route eigrp

## II. CẤU HÌNH OSPF

router ospf 1
 router-id 1.1.1.1
 network 192.168.10.0 0.0.0.255 area 0
 network 192.168.20.0 0.0.0.255 area 0

router ospf 1
 passive-interface default
 no passive-interface gig0/0

interface gig0/1
 ip ospf priority 200

show ip ospf neighbor
show ip ospf interface
show ip route ospf

## III. CẤU HÌNH iBGP

router bgp 65001
 bgp log-neighbor-changes
 neighbor 192.168.12.2 remote-as 65001
 network 10.10.10.0 mask 255.255.255.0

router bgp 65001
 bgp log-neighbor-changes
 neighbor 192.168.12.1 remote-as 65001
 network 20.20.20.0 mask 255.255.255.0

router bgp 65001
 neighbor 1.1.1.1 remote-as 65001
 neighbor 1.1.1.1 update-source loopback0

router bgp 65001
 neighbor 10.1.1.2 remote-as 65001
 neighbor 10.1.1.3 remote-as 65001

show ip bgp summary
show ip bgp neighbors
show ip bgp
show ip route bgp