# TỔNG HỢP LỆNH CISCO SWITCH (CƠ BẢN → NÂNG CAO)

## I. CÁC LỆNH CƠ BẢN

enable
configure terminal
exit
end
write memory / copy run start
reload
show version
show running-config
show startup-config
show interfaces
show interface status
show ip interface brief
show mac address-table
show vlan brief
show arp

## II. CẤU HÌNH VLAN VÀ IP CƠ BẢN

hostname SW-Core
interface vlan 1
 ip address 192.168.1.2 255.255.255.0
 no shutdown
exit
ip default-gateway 192.168.1.1

vlan 10
 name Sales
vlan 20
 name IT
exit

interface range fa0/1 - 10
 switchport mode access
 switchport access vlan 10
exit

interface gig0/1
 switchport trunk encapsulation dot1q
 switchport mode trunk
 switchport trunk allowed vlan 10,20,30

## III. BẢO MẬT CỔNG (PORT SECURITY)

interface fa0/1
 switchport mode access
 switchport access vlan 10
 switchport port-security
 switchport port-security maximum 2
 switchport port-security violation restrict
 switchport port-security mac-address sticky
show port-security interface fa0/1

## IV. SPANNING TREE (STP)

show spanning-tree
spanning-tree mode rapid-pvst
spanning-tree vlan 10 priority 4096
spanning-tree portfast
spanning-tree bpduguard enable

## V. ETHERCHANNEL (PORT CHANNEL)

interface range gi0/1 - 2
 channel-group 1 mode active
exit
interface port-channel 1
 switchport mode trunk
 switchport trunk allowed vlan 10,20
show etherchannel summary

## VI. LAYER 3 ROUTING

ip routing
interface vlan 10
 ip address 192.168.10.1 255.255.255.0
interface vlan 20
 ip address 192.168.20.1 255.255.255.0
ip route 0.0.0.0 0.0.0.0 192.168.1.1

## VII. DHCP SERVER TRÊN SWITCH L3

ip dhcp pool VLAN10
 network 192.168.10.0 255.255.255.0
 default-router 192.168.10.1
 dns-server 8.8.8.8
 lease 7
exit
ip dhcp excluded-address 192.168.10.1 192.168.10.10

## VIII. SSH VÀ BẢO MẬT ĐĂNG NHẬP

hostname SW-CORE
ip domain-name mylab.local
crypto key generate rsa modulus 2048
username admin privilege 15 secret MyPassword
line vty 0 4
 transport input ssh
 login local

## IX. CHẨN ĐOÁN VÀ GIÁM SÁT

show log
show controllers
show interface counters errors
show processes cpu
show memory statistics
show power inline
snmp-server community public RO
snmp-server community private RW
snmp-server location ServerRoom
snmp-server contact admin@company.local

## X. ACL, SYSLOG, NTP

ip access-list standard BLOCK_SSH
 deny host 192.168.10.50
 permit any
interface vlan 10
 ip access-group BLOCK_SSH in
logging 192.168.1.100
ntp server 192.168.1.10